ISOTOP SD

TENDERSPECIFICATION

Manufacturer information

Getzner Werkstoffe GmbH

Herrenau 5

A-6706 Bürs

Austria

www.getzner.com

Brand

Isotop® SD - Steel spring isolator

Product properties

| # Application | # For isolating structure-borne noise and vibrations |
| --- | --- |
| # Efficiency of isolation | >99% (referred to 50 Hz excitation frequency or 3000 rpm) |
| # Consisting of | - Two spring caps with M10 internal thread (blind rivet nut) - Cylindrical coil spring according to DIN EN 13906-1 - CDC-coated or zinc galvanised - Optionally with Isotop® FP/K or FP base plate and Isotop® NV height adjustment |
| # Natural frequency | # from 3.2 Hz = 192 rpm |
| # Maximum load bearing | # from 20 kg to 525 kg |
| # For the bearing of |  |
| # Total weight | kg |
| # Excitation frequency | Hz or rpm |

The detailed type selection for the elements is based on the weight distribution of the respective devices.

Offered product

ISOTOP SD from Getzner Werkstoffe GmbH or equal.

| # Quantity | pcs. |
| --- | --- |
| # Sum | € |